Invitation for supply and installation of filtration plant. BoQ.

| FILTER PLANT SPECIFICATIONS- |  | Quantity |
| --- | --- | --- |
| -Type- Manufactured in conformity with Approval- Diameter- Total Height Steel Type Pressure Rating Mini Test Pressure Gasket Type Nozzle Plate Nozzle Material Nozzle Number Vessel Air Vent -MULTI-MEDIA FILLINGS- Filter VALVES- PVC Ball valve PVC Diaphragm valve FLOW METER Variable Area Flow Meter BLOWER Motor Efficiency Class RAW/FEED PUMP Motor Efficiency Class Frequency converter RINSE PUMP Motor Efficiency Class Frequency converter CONTROL PANEL Harmonized Standards Breaker 24VDC LEVEL SWITCH Model Type | NS40 PED 2014/68/EU CE Marked Certified/Approved 400mm 1630 +/-10mm P265GH EN10028-2 6 Bar 11.4 Bar EPDM 70 PP-13 PP Type 7 P SIPRO TOP Hydro-anthracite H 1 Gravel-0 Media Gravel -3 Media Gravel-C supportive layer Gravel- A supportive layer 546 PVC-U *GF (DVGW,ABS,SEPRO,TSSA) 514 PVC-U *GF 335 PVC -U *GF Becker IE2 CRI/CRN IE3 YES IE3 YES Conformity of Electromagnetic Compatibility (2004/108/EC). EN 61000-6-2 & EN 61000-6-3 Schneider PULS Gmbh QFS30W2TPKO5 E.L.B | 1unit Set requirement Set requirement Set requirement Set requirement Set requirement Set requirement Set requirement |

Works at filtration site -

- SINTEX TANKS - PIPINGS - CONNECTIONS - Electrification - Plant house - Fencing

Conditions

1.Valid Trade  & Service License.

Submission of Vessel design with inter piping and valves.

Submission of Pump design.

P & I diagram required.

WARRANTY REQUIRED Certified warranty for 2 years on the Plant, Machinery & Electrical components